VT1000 64GB SD

VT1000 64GB SD

SD CARD SUPPORT

SD CARD SUPPORT

STEPS

- Check Current version of PC viewer and VT1000 Firmware

- Download the latest PC viewer 1.0.8.0 from (LINK) Support.visiontrack.com

- Download the latest VT1000 firmware 1.1.4 (LINK) Support.visiontrack.com

- Install PC viewer

- Copy firmware to SD card

- Insert SD card into VT1000 (Firmware upgrade)

- Leave recording for a few minutes then check SD card back in PC viewer to confirm firmware has applied

- Initialise SD card, check and change settings and put back into VT1000

1

- Check current firmware and PC viewer version

- Remove SD card from VT1000

- Insert into PC and open PC viewer software

- Click on

- Select the SD card

- Select the  icon

- This will show you the current PC viewer and Firmware installed on the VT1000

2

- If required, download the latest PC viewer 1.0.8.0 from (LINK) support.visiontrack.com

- Remove the old version from your PC and install the latest version.

3

- If required download the latest VT1000 firmware 1.1.4 (LINK) support.visiontrack.com

- Download the zip file and unzip the folder, move the folder (program) to the root of the VT1000 SD card.

4

- Firmware upgrade
(DO NOT TURN THE POWER OFF DURING THE UPGRADE AS THIS WILL CASUE THE UNIT TO FAIL)

- Put the SD card into the VT1000.
The VT1000 will flash red and blue at the same time for a few seconds and then reboot.

- Once complete the VT1000 will show the blue LED, press the panic button and leave the unit recording for a few minutes.

- Turn off the VT1000 and put the SD card back into the PC viewer.

- Follow step 1 to check the firmware has applied

5

- Initialise SD, check and change settings

- Click on

- Click on Init SD Card

- Select ‘Quick Format’ and remove the tick from ‘Backup Configuration File’ and click start

- Once initialised

- Change any settings you need to, PLEASE MAKE SURE THE TICK IS REMOVED FROM SD CARD AUTO FORMAT FEATURE

- Click on ‘OK’

- Select ‘OK’ to save current settings

- You can now remove the SD card and place this back into the VT1000.

support.visiontrack.com

support.visiontrack.com

Need Support?

Need Support?